24-HOUR DIGITAL CLOCK USING

8051 MICROCONTROLLER

& RTC DS1307 (I²C)

(S.Y. Electronics and Telecommunication Engineering)

Microcontroller And Applications Project

# Submitted by

Megh Lahoti - 612407111

Harsh Modak - 612407114

Varad Moghe - 612407115

Under the Guidance of

Dr. Ashwini Kulkarni

Department Of Electronics & Telecommunication Engineering

COEP Technological University,

Shivaji Nagar, Pune – 411005, Maharashtra

Table of Contents

Abstract

1.1  Background and Context

1.2  Problem Statement

1.3  Motivation

1.4  Project Objectives

Chapter 2: Theoretical Framework

2.1  The I²C Protocol

2.2  DS1307 Real-Time Clock IC

2.3  7447 BCD-to-Seven-Segment Decoder

2.4  Multiplexed Seven-Segment Display

Chapter 3: System Architecture and Design

3.1  System Block Diagram

3.2  Hardware Specifications

3.3  Interrupt Architecture

Chapter 4: Firmware Design and Implementation

4.1  Development Environment

4.2  Register Allocation

4.3  Subroutine Map

4.4  I²C Communication Flow

4.5  Time Set Mode — State Machine

Chapter 5: Proteus Simulation and Results

5.1  Simulation Setup

5.2  Normal Mode Operation

5.3  Time Set Mode Operation

5.4  I²C Waveform Verification

Chapter 6: Conclusion and Future Scope

6.1  Conclusion

6.2  Applications

6.3  Future Scope and Extensions

References

Appendix A: Assembly Language Source Code

A.1  Subroutine Index

A.2  Full Source Listing

# Abstract

This report presents the design and implementation of a 24-hour digital clock using the AT89C51 (8051-architecture) microcontroller and the DS1307 Real-Time Clock (RTC) IC. DS1307 communicates with the microcontroller via the I²C (Inter-Integrated Circuit) serial protocol at a standard mode clock frequency of 100 kHz. The DS1307 is configured to produce a 1 Hz square wave output on its SQW pin, which is wired to the external interrupt line (INT1) of the 8051. This interrupt-driven architecture ensures the microcontroller reads and refreshes the display precisely once per second without requiring software managed timing loops.

The time information (hours and minutes) is retrieved from the DS1307 over I²C, stored in internal 8051 registers, and then driven to a four-digit seven-segment display (Groove 4-Digit module) through a 7447 BCD-to-seven-segment decoder operating in a multiplexed configuration. The entire firmware is written in 8051 Assembly Language and was developed in Keil µVision 5. Circuit simulation and timing verification were performed in Proteus Simulation Software using the AT89C51 device model. A Time Set Mode block is implemented using two switches, allowing manual RTC time adjustment without requiring a programming interface.

# Chapter 1: Introduction

## 1.1 Background and Context

Real-time clock (RTC) ICs are devices that maintain accurate time and calendar information independently of the host processor. They incorporate a low frequency crystal oscillator (typically 32.768 kHz), a prescaler chain and battery backed registers that preserve time data even when the main system is powered off. Offloading timekeeping to a dedicated RTC rather than relying on software timer loops eliminates the accumulated error introduced by interrupt latency and program execution overhead.

The DS1307 is one of the most widely used RTC ICs in educational and embedded system designs. It communicates over I²C, providing a simple two-wire (SDA / SCL) interface that is easily implemented on the 8051's general purpose I/O ports. The device maintains seconds, minutes, hours, day, date, month, and year registers in Binary Coded Decimal (BCD) format and also features a programmable square-wave output, a key feature exploited in this project for interrupt generation.

## 1.2 Problem Statement

A common challenge in microcontroller-based clock designs is maintaining long term timekeeping accuracy while simultaneously executing display refresh, user input scanning, and I/O communication tasks. If the microcontroller manages time via software delays, any interrupt service routine or blocking operation introduces drift. This project addresses that problem by transferring timekeeping responsibility entirely to the DS1307 hardware, reducing the 8051's role to reading, formatting, and displaying time, tasks that tolerate occasional latency.

## 1.3 Motivation

This project was undertaken to develop practical understanding in three core areas of embedded systems: I²C serial communication protocol implementation, interrupt driven firmware architecture, and multiplexed seven segment display driving. Implementing I²C in Assembly Language, rather than relying on a hardware I²C peripheral builds a thorough understanding of the protocol's start condition, stop condition, data framing, acknowledgement, and clock stretching mechanisms. Simulation in Proteus prior to hardware assembly mirrors professional pre-silicon verification workflows.

## 1.4 Project Objectives

- Implement I²C master communication routines in 8051 Assembly to configure and read the DS1307 RTC.

- Configure the DS1307 SQW output at 1 Hz and use it as the INT1 interrupt source for precise one second timekeeping intervals.

- Drive a 4 digit seven-segment display using the 7447 BCD decoder in a time-multiplexed configuration, cycling through all four digits at an imperceptible refresh rate.

- Implement a two-switch Time Set Mode for manual time adjustment without a programming interface.

- Verify full design correctness through Proteus simulation before hardware implementation.

# Chapter 2: Theoretical Framework

## 2.1 The I²C Protocol

I²C (Inter-Integrated Circuit) is a synchronous, half-duplex, multi-master serial communication protocol developed by Philips Semiconductor. It uses two bidirectional open drain lines: SDA (Serial Data) and SCL (Serial Clock). Multiple devices can share the same bus, each identified by a unique 7-bit or 10-bit address. Standard mode I²C supports a clock frequency of up to 100 kHz; fast mode extends this to 400 kHz.

A transaction is framed by a START condition (SDA transitions LOW while SCL is HIGH) and a STOP condition (SDA transitions HIGH while SCL is HIGH). Data is transferred MSB-first in 8-bit frames, each followed by an Acknowledge (ACK) or Not-Acknowledge (NAK) bit driven by the receiver. The 8051 does not have a hardware I²C peripheral, so all timing and signaling is implemented in software using General Purpose I/O (GPIO) bit manipulation, commonly called bit-banging.

| Signal | Direction | Description |
| --- | --- | --- |
| SDA | Bidirectional (OD) | Serial data line carries address and data bytes MSB first |
| SCL | Master drives (OD) | Serial clock line generated by I²C master (8051) |
| START | Master asserts | SDA falls while SCL is HIGH initiates a transaction |
| STOP | Master asserts | SDA rises while SCL is HIGH terminates a transaction |
| ACK | Receiver drives | Receiver pulls SDA LOW during 9th clock acknowledges |
| NAK | Receiver drives | Receiver leaves SDA HIGH during 9th clock not acknowledged |

Table 1: I²C bus signals and their functions.

## 2.2 DS1307 Real-Time Clock IC

The DS1307 is a low-power, full binary-coded decimal (BCD) clock and calendar IC with 56 bytes of NV SRAM. It is powered from a primary 5 V supply and includes a backup battery input (VBAT) for continued operation during power failures. The device operates as an I²C slave with the fixed 7-bit address 0x68 (1101000 in binary).

| Address | Register | Format | Description |
| --- | --- | --- | --- |
| 00h | Seconds | BCD (00–59) | Bit 7 = Clock Halt (CH). Set CH=0 to enable oscillator. |
| 01h | Minutes | BCD (00–59) | Minutes value in BCD. |
| 02h | Hours | BCD (00–23) | Bit 6 = 12/24hr mode. Bit 5 = AM/PM in 12hr mode. |
| 03h | Day | BCD (01–07) | Day of the week (1=Sunday). |
| 04h | Date | BCD (01–31) | Day of the month. |
| 05h | Month | BCD (01–12) | Month. |
| 06h | Year | BCD (00–99) | Year (last two digits). |
| 07h | Control | Bitmap | Bit 4 = SQW enable; Bits 1:0 = SQW frequency (00=1Hz). |

Table 2: DS1307 internal register map (addresses 00h–07h).

## 2.3 7447 BCD to Seven Segment Decoder

The SN7447 is a TTL-family BCD-to-seven-segment decoder/driver designed for direct LED common-anode displays. It accepts a 4 bit BCD input (A, B, C, D where A is the LSB) and drives seven active-LOW outputs (a–g) corresponding to the segments of a seven-segment display. Additional control pins include:

- LT (Lamp Test, active LOW): Forces all segments ON when asserted — useful for display testing.

- RBI (Ripple Blanking Input, active LOW): Suppresses leading-zero display on the most-significant digit.

- BI/RBO (Blanking Input / Ripple Blanking Output): Blanks the digit when asserted LOW; also propagates blanking signal to adjacent digit drivers in a multi-digit cascade.

In this design, the 7447 operates with LT and RBI tied inactive. The 4-bit BCD code is driven from the 8051 Port 2 (P2.0–P2.3), and the decoded segment outputs are routed to the Groove 4-digit display module. Only one digit is enabled at a time through digit-select lines controlled by the 8051, achieving time-division multiplexing.

## 2.4 Multiplexed Seven-Segment Display

Driving four seven-segment digits simultaneously would require 4 × 7 = 28 I/O lines. Multiplexing reduces this to 7 segment lines + 4 digit-select lines = 11 lines. The microcontroller cycles through each digit at a rate exceeding ~50 Hz (so the human eye perceives all digits as continuously lit due to persistence of vision). In each cycle, the digit-select line for one digit is asserted LOW while the BCD code for that digit's value is presented to the 7447. The cycle then moves to the next digit. Since only one digit is driven at a time, the current through each LED segment is higher than in a static drive scheme; segment resistors are sized accordingly.

# Chapter 3: System Architecture and Design

## 3.1 System Block Diagram

The system is composed of five functional blocks interconnected as follows:

- Power Supply: +5 V regulated supply powering the 8051, 7447, and display. A 3 V CR2032 coin cell backs the DS1307 VBAT pin (not depicted in simulation).

- 8051 (AT89C51) Microcontroller: Central processing unit. Runs the firmware, manages I²C communication, handles interrupts, and drives the display.

- DS1307 RTC: Maintains accurate time in BCD registers. Generates a 1 Hz interrupt signal on its SQW pin.

- 7447 + 4-Digit 7-Segment Display: Decodes BCD to segment signals and renders the time digits.

- User Input (Switch A & Switch B): Two switches enabling manual time adjustment via the Time Set Mode.

Figure 1: Proteus 8.9 simulation of complete 24-hour digital clock circuit (AT89C51, DS1307, 7447, 4-digit display).

## 3.2 Hardware Specifications

| Parameter | Value / Description |
| --- | --- |
| Microcontroller | AT89C51 (8051 architecture, 4 KB Flash, 128 B RAM) |
| Crystal (8051) | 11.0592 MHz (standard for exact 9600 baud; provides precise timer intervals) |
| RTC IC | DS1307 (Maxim / Analog Devices) |
| RTC Crystal | 32.768 kHz watch crystal with 12.5 pF load capacitors (X2, C1, C2) |
| RTC Backup | 3 V CR2032 coin cell on VBAT pin |
| I²C Interface | Bit-banged on 8051 P1.6 (SDA) and P1.7 (SCL) |
| I²C Speed | Standard mode — 100 kHz (software controlled) |
| Display | Groove 4-Digit Common-Anode 7-Segment Display |
| BCD Decoder | SN7447 BCD-to-7-segment decoder/driver |
| Display Drive | Multiplexed — 4-digit time-division at ~200 Hz refresh |
| Supply Voltage | 5 V DC (regulated) |
| Switch A (INT1) | SPDT — set time / advance digit (interrupt-based) |
| Switch B | SPDT — selects interrupt operation mode |
| LED indicator | SET LED via R5 (100k pull-up) + R8 (330Ω) — active in Time Set Mode |
| Simulation Tool | Proteus 8.9 (AT89C51 model) |
| Firmware IDE | Keil µVision 5 — 8051 Assembly Language |

Table 3: Hardware specifications and design parameters.

## 3.3 Interrupt Architecture

The firmware is built around an interrupt-driven architecture. Two interrupt sources are used:

- INT1 (External Interrupt 1 — Pin P3.3): Triggered by the DS1307 SQW 1 Hz output. On each falling edge, the ISR calls UPDATETIME to read the RTC over I²C and refresh the display registers. Additionally, if Switch B is LOW, INT1 interprets the trigger as a time-set advance command.

- The main loop continuously calls the DISPLAY subroutine to cycle through all four digits, ensuring the display remains lit while the processor idles between interrupts.

# Chapter 4: Firmware Design and Implementation

## 4.1 Development Environment

- IDE: Keil µVision 5 — industry-standard IDE for 8051/ARM embedded development.

- Language: 8051 Assembly Language — chosen to develop low-level register and I/O mastery, and for precise control over instruction cycle timing required by the bit-banged I²C routines.

- Simulator: Proteus 8.9 — schematic capture and mixed-signal SPICE simulation including AT89C51 and DS1307 device models.

- Hex output: Keil generates an Intel HEX file loaded into the AT89C51 program memory in Proteus for co-simulation.

## 4.2 Register Allocation

The following 8051 working registers (bank 0) are assigned fixed roles throughout the firmware to avoid register conflicts between the main loop, ISR, and subroutines:

| Register | Name / Role | Description |
| --- | --- | --- |
| R1 | ACC Backup | Saves Accumulator (A) before subroutine calls that may corrupt it. |
| R2 | MICRO_DELAY | Loop counter for the micro-delay subroutine — controls I²C bit timing. |
| R3 | TSM Temp | Temporary storage inside the SETTIMEMODE subroutine. |
| R4 | Hours Reg | Stores current hours as 0x(MSB)(LSB) — BCD nibble pairs. |
| R5 | Minutes Reg | Stores current minutes as 0x(MSB)(LSB) — BCD nibble pairs. |
| R6 | TSM State | Tracks which digit the Time Set Mode is currently editing (0x00–0x04). |

Table 4: 8051 register allocation map.

Time Set Mode states stored in R6:

| R6 Value | Mode |
| --- | --- |
| 0x00 | Normal Mode — display updates from RTC every second |
| 0x01 | MSB of Hours being set |
| 0x02 | LSB of Hours being set |
| 0x03 | MSB of Minutes being set |
| 0x04 | LSB of Minutes being set |

Table 5: Time Set Mode state machine (R6 register).

## 4.3 Subroutine Map

| Address | Label | Function |
| --- | --- | --- |
| 0150H | SETTIMEMODE | Called by INT1 ISR. Implements the time-set state machine using R6. Reads Switch B to decide whether to toggle the current digit or advance to the next. |
| 0260H | DISPLAY | Drives the 7447 BCD decoder in multiplex mode. Outputs BCD nibbles of R4 (hours) and R5 (minutes) to P2, cycling digit-select lines. |
| 0300H | UPDATETIME | Reads DS1307 hours (02h) and minutes (01h) registers over I²C and updates R4 and R5. Called every second by INT1 ISR in Normal Mode. |
| 02F0H | MICRO_DELAY | Software delay loop using R2. Provides bit-period timing for I²C clock generation (~5 μs half-period at 11.0592 MHz). |
| 0400H | RTCTIMEUPDATE | Writes updated time from R4/R5 back to DS1307 minutes (01h) and hours (02h) registers via I²C. Called when exiting Time Set Mode. |
| 0510H | RSTART | I²C Repeated START condition — SDA goes HIGH then LOW while SCL is HIGH, without issuing a STOP first. Used for register pointer repositioning. |
| 0520H | STARTC | I²C START condition — pulls SDA LOW while SCL is HIGH, then pulls SCL LOW to begin data phase. |
| 0530H | STOP | I²C STOP condition — releases SDA HIGH while SCL is HIGH to signal end of transaction. |
| 0540H | SEND | Transmits 8 bits from Accumulator MSB-first over SDA, clocking each bit with SCL pulses. Waits for ACK. |
| 0600H | ACK | Master-to-Slave Acknowledge — pulls SDA LOW during 9th clock pulse to acknowledge received byte. |
| 0610H | NAK | Master-to-Slave Not-Acknowledge — releases SDA HIGH during 9th clock pulse. Used after reading the last byte to signal no more data needed. |
| 0620H | RECV | Receives 8 bits from SDA MSB-first into Accumulator, clocking each bit with SCL. Calls ACK or NAK as appropriate. |

Table 6: Firmware subroutine address map with functional descriptions.

## 4.4 I²C Communication Flow

The I²C read sequence for retrieving one DS1307 register follows the standard "Write address pointer, then Read" transaction pattern:

Write phase (set register pointer):

STARTC                         ; Assert START condition

MOV A, #0xD0                   ; DS1307 write address (0x68 << 1 | W=0)

CALL SEND                       ; Send slave address + W bit, check ACK

MOV A, #0x01                   ; Register pointer -> Minutes register

CALL SEND                       ; Send register address

Read phase (retrieve data):

RSTART                          ; Repeated START (no STOP between phases)

MOV A, #0xD1                   ; DS1307 read address (0x68 << 1 | R=1)

CALL SEND                       ; Send slave address + R bit, check ACK

CALL RECV                       ; Receive Minutes byte -> A, send ACK

MOV R5, A                       ; Store minutes in R5

CALL RECV                       ; Receive Hours byte -> A, send NAK

MOV R4, A                       ; Store hours in R4

STOP                            ; Release bus

The write sequence for RTCTIMEUPDATE follows the same pattern but sends R4 and R5 back to the DS1307 registers after each SEND, completing the full RTC update cycle.

## 4.5 Time Set Mode — State Machine

The Time Set Mode allows the user to manually adjust the RTC time without a programmer. It is controlled by two switches and implemented as a state machine stored in register R6.

Figure 2: Time Set Mode switch operation guide

| Switch Combination | Switch B State | Action |
| --- | --- | --- |
| Toggle A ON→OFF | ON (HIGH) | Enter Time Set Mode (R6: 0x00 → 0x01). SET LED turns ON. |
| Toggle A ON→OFF | OFF (LOW) | Toggle (increment) the currently selected BCD digit. |
| Toggle A ON→OFF | ON (HIGH) | Save current digit and advance to next (R6 increments). Previous digit is written to RTC. |
| Toggle A ON→OFF (twice) | ON | After last digit (R6=0x04), two consecutive advances exit Time Set Mode. Full time written to DS1307 via RTCTIMEUPDATE. LED turns OFF. |

Table 7: Time Set Mode switch combination reference.

Important: Switch A is the interrupt source (INT1). The state of Switch B at the moment Switch A is toggled determines the operation performed. Always set Switch B to the correct state before actuating Switch A. In a physical hardware implementation, a debounce circuit (RC filter + Schmitt trigger, or software debounce) must be used on Switch A to prevent multiple spurious interrupts from a single press.

# Chapter 5: Proteus Simulation and Results

## 5.1 Simulation Setup

The complete schematic was drawn in Proteus 8.9. The AT89C51 device model was loaded with the Intel HEX file generated by Keil µVision. The DS1307 component in Proteus supports I²C slave simulation, allowing the 8051 firmware to read and write RTC registers in real time during simulation. The Groove 4-digit display was represented using the GROVE4DIGIT component available in the Proteus library.

- Crystal X1 (8051): 11.0592 MHz — selected to produce exact timer counts for baud-rate generation and to simplify delay calculations.

- Crystal X2 (DS1307): 32.768 kHz — mandatory for DS1307 oscillator operation.

- Capacitors C1, C2 (33 pF): Load capacitors on the 8051-crystal circuit for stable oscillation.

- Resistors R1, R2, R3: I²C bus pull-up resistors on SDA and SCL lines, plus a pull-up on the SQW interrupt line.

## 5.2 Normal Mode Operation

In Normal Mode (R6 = 0x00), the 8051 main loop continuously calls DISPLAY to refresh all four digits. Each second, the DS1307 SQW output (1 Hz) triggers INT1. The ISR calls UPDATETIME, which reads the DS1307 registers over I²C and updates R4 (hours) and R5 (minutes). The display then reflects the new time on the next refresh cycle.

Figure 3: Proteus simulation of Normal Mode. 4-digit display shows current 24-hour time retrieved from DS1307 over I²C.

## 5.3 Time Set Mode Operation

Activating Time Set Mode (Switch B HIGH, then toggle Switch A) sets R6 to 0x01 and illuminates the SET LED. The display continues to show the time being edited. Each toggle of Switch A with Switch B LOW increments the selected BCD digit by one (wrapping at the appropriate maximum: 2 for MSB of hours in 24hr mode, 3 for MSB of minutes, 9 for LSB digits). Toggle Switch A with Switch B HIGH to save the current digit and advance to the next.

Figure 4: Proteus simulation of Time Set Mode active. SET LED is illuminated; Switch A/B being used to adjust time digits.

## 5.4 I²C Waveform Verification

The I²C SDA and SCL signals were monitored using the Proteus logic analyzer tool to verify correct protocol implementation. Key observations:

- START condition confirmed: SDA falls while SCL is HIGH before each transaction.

- Data bits sampled correctly on SCL rising edge; transitions occur while SCL is LOW.

- ACK bits from DS1307 visible as SDA pulled LOW on the 9th clock of each byte.

- STOP condition confirmed: SDA rises while SCL is HIGH at end of each transaction.

Figure 5: I²C (SDA and SCL) captured in Proteus.

# Chapter 6: Conclusion and Future Scope

## 6.1 Conclusion

This project successfully demonstrated a complete interrupt-driven 24-hour digital clock using the AT89C51 microcontroller and DS1307 RTC IC. The I²C protocol was implemented entirely in 8051 Assembly Language using bit-banging on GPIO pins, achieving standard-mode (100 kHz) compliant communication verified by Proteus simulation. The DS1307's 1 Hz SQW output provided a hardware-accurate interrupt source, eliminating software timing drift entirely.

The multiplexed 7447 + 4-digit seven-segment display reliably rendered hours and minutes, and the two-switch Time Set Mode provided an intuitive interface for manual RTC time adjustment. All firmware subroutines were verified in simulation before hardware implementation. The design met all stated objectives: accurate timekeeping, correct I²C communication, functional display multiplexing, and user-adjustable time setting.

## 6.2 Applications

- Embedded alarm clocks and timer circuits in home automation.

- Industrial process timing and scheduling controllers.

- Data loggers requiring accurate timestamping of sensor readings.

- Educational embedded systems kits for demonstrating I²C and interrupt-driven design.

- Battery-backed calendar modules for microcontroller systems lacking an on-chip RTC.

## 6.3 Future Scope and Extensions

- Alarm Functionality: Add a third register (R7 = Alarm time) and compare it with the current time at each INT1 interrupt. Assert a buzzer or relay output when times match.

- Date Display: Read DS1307 registers 03h–06h to display day/month/year on additional display pages, toggled by a mode button.

- UART Debug Output: Use the 8051 UART (P3.0/RXD, P3.1/TXD) to transmit the current time as ASCII over RS-232 or USB-to-UART for PC logging.

- LCD Interface: Replace the 7-segment display with a 16×2 LCD (HD44780) driven via a 4-bit interface on Port 1, enabling display of date, day of the week, and alarm status.

- Higher-Level Language Port: Re-implement the firmware in C (Keil C51) to improve maintainability and portability, retaining Assembly only for the time-critical I²C bit-bang routines.

- PCB Design: Transfer the validated breadboard circuit to a two-layer PCB with proper decoupling capacitors, ground planes, and crystal guard rings to minimise EMI.

- DS3231 Upgrade: Replace DS1307 with DS3231, which integrates a temperature-compensated crystal oscillator (TCXO) for ±2 ppm accuracy (±1 minute per year) and eliminates the external crystal requirement.

# References

[1]  Intel Corporation, MCS-51 Microcontroller Family User’s Manual, Intel, 1994.

[2]  Maxim Integrated (Analog Devices), DS1307 64 × 8 Serial Real-Time Clock Datasheet, Rev. 10, 2015. [Online]. Available: https://www.analog.com/en/products/ds1307.html

[3]  Texas Instruments, SN7447A BCD-to-Seven-Segment Decoder/Driver Datasheet, SDLS126, 1988 (revised 2003). [Online]. Available: https://www.ti.com/product/SN7447A

[4]  NXP Semiconductors, I²C-Bus Specification and User Manual, Rev. 7.0, UM10204, 2021. [Online]. Available: https://www.nxp.com/docs/en/user-guide/UM10204.pdf

[5]  Atmel Corporation (Microchip), AT89C51 8-bit Microcontroller Datasheet, Rev. 1487Z-8051-07/10. [Online]. Available: https://www.microchip.com/en-us/product/at89c51

[6]  Labcenter Electronics, Proteus Design Suite 8 — VSM Simulation User Manual, 2023. [Online]. Available: https://www.labcenter.com

[7]  Arm Keil, Keil µVision 5 for 8051 — Getting Started Guide, 2023. [Online]. Available: https://www.keil.com/support/docs/220.htm

[8]  M. A. Mazidi, J. G. Mazidi, and R. D. McKinlay, The 8051 Microcontroller and Embedded Systems, 2nd ed. Pearson, 2006.

# Appendix A: Assembly Language Source Code

The complete 8051 Assembly Language firmware for the 24-hour digital clock is listed below. The code was assembled using Keil µVision 5 and the resulting Intel HEX file was loaded into the AT89C51 in Proteus 8.9 for simulation.

| Property | Detail |
| --- | --- |
| File Name | mca.asm |
| Assembler | Keil µVision 5 — A51 Macro Assembler |
| Target Device | AT89C51 (8051 compatible) |
| Crystal | 11.0592 MHz |
| Language | 8051 Assembly (Intel MCS-51 instruction set) |
| Total Subroutines | 12 (see Table 6 for address map) |

Table A1: Source file properties.

mca.asm: 8051 Assembly Source Code

;REGISTERS USED: ;R1 -> ACCUMULATOR BACKUP ;R2 -> MICRO_DELAY FUNCTION ;R3 -> TEMPORARY STORAGE IN TIMESETMODE SUBROUTINE ; MSB LSB ;R4 -> HOURS REGISTER ----- 0 x X X ;R5 -> MINUTES REGISTER ----- 0 x X X ;;;;R6 -> TIME SET REGISTER [COMMAND] ; 0x00: Normal Mode ; 0x01: MSB HOUR SET ; 0x02: LSB HOUR SET ; 0x03: MSB MINUTES SET ; 0x04: LSB MINUTES SET ;SUBROUTINE ADDRESSES ;(150H)SETTIMEMODE -> INT1 CALLS THIS ADDRESS FOR MANUALLY SETTING TIME ;(260H)DISPLAY -> 8051 DRIVING 7447 BCD TO 7SEG DISPLAY DECODER IN MULTIPLEX MODE ;(300H)UPDATETIME -> CALL TO UPDATE 8051 INTERNAL TIME REGISTERS R4 AND R5 ;(2F0)MICRO_DELAY -> CALL A SMALL DELAY ;(400H)RTCTIMEUPDATE -> CALLED WHILE MANUALY UPDATING TIME IN RTC CHIP ;(510H)RSTART -> I2C RESTART CONDITION ;(520H)STARTC -> I2C START CONDITION ;(530H)STOP -> I2C STOP CONDITION ;(540H)SEND -> I2C SEND DATA ;(600H)ACK -> I2C ACKNOWLEDGEMENT (M TO S) ;(610H)NAK -> I2C N-ACKNOWLEDGEMENT (M TO S) ;(620H)RECV -> I2C RECEIVE DATA ;*************************************** ;PORTS USED FOR I2C COMMUNICATION ;*************************************** SDA EQU P1.7 SCL EQU P1.6 ;*************************************** ;PORT PIN USED FOR SETTING TIME TOGGLE EQU P3.4 ; SWITCH B SETTIMELED EQU P3.5 ;LED TO INDICATE IF 8051 IS IN TIMESET MODE ORG 0000H LJMP START //-----------------EX0 RTC INTERRUPT------------------------ ORG 0003H LCALL TIMEUPDATE RETI //---------------------------------------------------------- //------------EX1 SETTIME INTERRUPT BY SWITCH A------------- ORG 0013H LCALL SETTIMEMODE RETI //---------------------------------------------------------- ORG 100H START: MOV R4, #00H ;HOURS DEFAULT 00H MOV R5, #00H ;MINUTES DEFAULT 00H MOV R6, #00H ;NORMAL MODE //-----------------1HZ SQ WAVE O/P CONFIGURE---------------------- ;SEND START CONDITION LCALL STARTC ;SEND SLAVE ADDRESS MOV A,#11010000B ;I2C ID 1101000X WHERE X = 0 FOR WRITE OPERATION LCALL SEND ;SLAVE ACK SKIPPED STORED IN C ;SEND DATA MOV A, #00000111B ; SQUARE O/P REGISTER SELECT LCALL SEND MOV A, #10H ;INIT 1HZ SQUARE WAVE O/P LCALL SEND LCALL STOP //------------8051 INTERRUPT SET---------------- MOV IE, #85H ;ENABLE INT0 AND INT1 INTERRUPT PRIORITY ONLY MOV TCON, #05H ;ENABLE ENTERNAL INTERRUPT IN EDGE TRIGERRED MODE //-------------------------------------------- LJMP DISPLAY ORG 150H SETTIMEMODE: CJNE R6, #05H, OPTION ;CHECK WHICH MODE 8051 WAS OPERATING IN ;---------------------Return to normal mode----------------------------- MOV R6, #00H LCALL RTCTIMEUPDATE SETB EX0 ;ENABLE RTC INTERRUPTING 8051 SETB SETTIMELED ;DISABLE SET TIME INDICATOR RET ;-----------------------Time set Mode----------------------------------- OPTION: JNB TOGGLE, NEXTBIT ;IF SWTICH B IS ON, GO TO NEXT TIME BIT ;--------------------------------------------------- CJNE R6, #01H, SETHRLSB ;CHECK WHICH TIME BIT IS BEING SET MOV A, R4 ;COPY CURRENT HOUR DATA IN R4 ANL A, #0F0H ;IGNORE LOWER NIBBLE ADD A, #10H ;ADD 1 TO UPPER NIBBLE CJNE A, #30H, SKIPHRMSBRESET ;HOUR MSB CAN BE ONLY BETWEEN 0 - 2 [24HR CLOCK] MOV A, #00H SKIPHRMSBRESET: MOV R4, A RET ;--------------------------------------------------- SETHRLSB: CJNE R6, #02H, SETMINMSB ;CHECK WHICH TIME BIT IS BEING SET MOV A, R4 ANL A, #0F0H MOV R3, A ;FOR BACKING UP THE HOUR MSB BIT SET ALREADY MOV A, R4 ANL A, #0FH INC A CJNE R3, #20H, CHKFORHRMSB2 ;CHECK IS THE HOUR MSB IS 1 OR 2 CJNE A, #04H, SKIPHRLSBRESET ;IF HOUR MSB IS 2, HOUR LSB CAN BE BETWEEN 0 - 3 MOV A, #00H JMP SKIPHRLSBRESET CHKFORHRMSB2: CJNE A, #0AH, SKIPHRLSBRESET ;IF HOUR MSB IS 0 OR 1, HOUR LSB CAN BE BETWEEN 0 - 9 MOV A, #00H SKIPHRLSBRESET: ORL A, R3 MOV R4, A RET ;-------------------------------------------------- SETMINMSB: CJNE R6, #03H, SETMINLSB MOV A, R5 ANL A, #0FH MOV R3, A MOV A, R5 ANL A, #0F0H ADD A, #10H CJNE A, #60H, SKIPMINMSBRESET ;IF MIN MSB CAN BE BETWEEN 0 - 5 MOV A, #00H SKIPMINMSBRESET: ORL A, R3 MOV R5, A RET ;------------------------------------------------- SETMINLSB: CJNE R6, #04H, RETURNMINLSB MOV A, R5 ANL A, #0F0H MOV R3, A MOV A, R5 ANL A, #0FH INC A CJNE A, #0AH, SKIPMINLSBRESET ;MIN LSB CAN BE BETWEEN 0 - 9 MOV A, #00H SKIPMINLSBRESET: ORL A, R3 MOV R5, A RETURNMINLSB: RET ;---------------------------- NEXTBIT: INC R6 ; INCREMENT MODE [NORMAL MODE 00H, HOUR MSB 01H, HOUR LSB 02H, MIN MSB 03H, MIN LSB 04] ;WHEN R6 > 04H NEXT EX1 INTERRUPT MAKES R6 -> 00H AND 8051 COMES OUT OF TIME SET MODE CLR EX0 ;DISABLE RTC INTERRUPTING 8051 CLR SETTIMELED ;ENABLE SET TIME INDICATOR RET ORG 260H DISPLAY: //-------HOUR---------- //---MSB--- HOURMSB: MOV A, R4 SWAP A ANL A, #03H ; 0000 00XX ORL A, #10H ; ENABLE ONLY HOUR MSB DISPLAY MOV P2, A LCALL MICRO_DELAY CJNE R6, #01H, HOURLSB ;IF IN TIME SET MODE, ONLY SHOW THE SETTING TIME ON 7SEG DISPLAY SJMP DISPLAY //---LSB--- HOURLSB: MOV A, R4 ANL A, #0FH ORL A, #20H ; ENABLE ONLY HOUR LSB DISPLAY MOV P2, A LCALL MICRO_DELAY CJNE R6, #02H, MINMSB ;IF IN TIME SET MODE, ONLY SHOW THE SETTING TIME ON 7SEG DISPLAY SJMP DISPLAY //-------MIN---------- //---MSB--- MINMSB: MOV A, R5 SWAP A ANL A, #0FH ORL A, #40H ; ENABLE ONLY MIN MSB DISPLAY MOV P2, A LCALL MICRO_DELAY CJNE R6, #03H, MINLSB ;IF IN TIME SET MODE, ONLY SHOW THE SETTING TIME ON 7SEG DISPLAY SJMP DISPLAY //---LSB--- MINLSB: MOV A, R5 ANL A, #0FH ORL A, #80H ; ENABLE ONLY MIN LSB DISPLAY MOV P2, A LCALL MICRO_DELAY SJMP DISPLAY ORG 2F0H ; 5US DELAY MICRO_DELAY: MOV R2, #05H HERE: DJNZ R2, HERE RET ;***************************************** ; WRITE TO SLAVE DEVICE WITH SLAVE ADDRESS 1101000XB ;***************************************** ORG 300H TIMEUPDATE: //---------------UPDATE TIME REGISTERS----------------------------- //--------REGISTER POINTER INITIALIZE---------------- ; SEND START CONDITION LCALL STARTC ; SEND SLAVE ADDRESS MOV A,#11010000B ;I2C ID 1101000X WHERE X = 0 FOR WRITE OPERATION LCALL SEND MOV A, #01H ;INIT RTC POINTER TO MINUTES REGISTER LCALL SEND LCALL STOP //--------READ MIN AND HOURS DATA-------------------- ; SEND START CONDITION LCALL STARTC ; SEND SLAVE ADDRESS WITH READ BIT SET MOV A,#11010001B ;I2C ID 1101000X WHERE X = 1 FOR READ OPERATION LCALL SEND ; READ ONE BYTE LCALL RECV MOV R5, A ;UPDATE MINUTES REGISTER ; SEND ACK LCALL ACK ; READ LAST BYTE LCALL RECV MOV R4, A ;UPDATE MINUTES REGISTER ; SEND NAK FOR LAST BYTE TO INDICATE ; END OF TRANSMISSION LCALL NAK ; SEND STOP CONDITION LCALL STOP MOV A, R1 ;RESTORE ACCUMULATOR DATA RET ORG 400H RTCTIMEUPDATE: ;SEND START CONDITION LCALL STARTC ;SEND SLAVE ADDRESS MOV A,#11010000B ;I2C ID 1101000X WHERE X = 0 FOR WRITE OPERATION LCALL SEND ;SLAVE ACK SKIPPED STORED IN C ;SEND DATA MOV A, #00000000B ; SECONDS REGISTER SELECT LCALL SEND MOV A, #00H ; MAKE SECONDS REGISTER 00 LCALL SEND MOV A, R5 ;UPDATE RTC MINUTES REGISTER LCALL SEND MOV A, R4 ;UPDATE RTC HOURS REGISTER ORL A, #80H ;SET 24 HR BIT IN RTC FOR 24 HR OPERATION MODE LCALL SEND LCALL STOP RET ;**************************************** ;RESTART CONDITION FOR I2C COMMUNICATION ¦ [NOT USED] ;**************************************** ORG 510H RSTART: CLR SCL SETB SDA SETB SCL CLR SDA RET ;**************************************** ;START CONDITION FOR I2C COMMUNICATION ;**************************************** ORG 520H STARTC: SETB SCL CLR SDA CLR SCL RET ;***************************************** ;STOP CONDITION FOR I2C BUS ;***************************************** ORG 530H STOP: CLR SCL CLR SDA SETB SCL SETB SDA RET ;***************************************** ;SENDING DATA TO SLAVE ON I2C BUS ;***************************************** ORG 540H SEND: MOV R1, A ;BACKUP ACCUMULATOR DATA MOV R7,#08 BACK: CLR SCL RLC A MOV SDA,C SETB SCL DJNZ R7,BACK CLR SCL SETB SDA SETB SCL MOV C, SDA CLR SCL MOV A, R1 ;RESTORE ACCUMULATOR DATA RET ;***************************************** ;ACK AND NAK FOR I2C BUS ;***************************************** ORG 600H ACK: CLR SDA SETB SCL CLR SCL SETB SDA RET ORG 610H NAK: SETB SDA SETB SCL CLR SCL SETB SCL RET ;***************************************** ;RECEIVING DATA FROM SLAVE ON I2C BUS ;***************************************** ORG 620H RECV: MOV R1, A ;BACKUP ACCUMULATOR DATA, RESTORED AT THE END OF TIMEUPDATE CALL SUBROUTINE MOV R7,#08 BACK2: CLR SCL SETB SCL MOV C,SDA RLC A DJNZ R7,BACK2 CLR SCL SETB SDA RET END